Name:					  Date:

Class:					Teacher:

## Lesson 5: How Do Robots Think?

Activity 5– KISS - Keep it Simple with Subroutines

Put an X in the box for each item as you complete the parts of the activity:

Pre-Activity Check List:

|  | - Fresh/Charged Batteries installed into the Scribbler 2 robot. |
| --- | --- |
|  | - Scribbler Connected to your computer via USB cable and USB to Serial. |
|  | - Clear area to work with the Robot still connected to your computer. |
|  | - Review the Creating/Calling Subroutines Section of the Help Documentation |

Activity Checklist:

|  | A: Calling for SOS |
| --- | --- |
|  | - Open Scribbler Program Maker Software |
|  | - Locate the Make a New Subroutine Tile |
|  | - Select a subroutine color and insert it into the program |
|  | - Insert a Light Tile, a Timer, another Light Tile and a longer Timer Tile into the yellow subroutine |
|  | - Create an orange subroutine |
|  | - Insert a Light Tile, a Timer, another Light Tile and a longer Timer Tile into the orange subroutine |
|  | - Create a Red Subroutine |
|  | - Insert 3 calls to the yellow subroutine into the red subroutine |
|  | - Create a Blue Subroutine |
|  | - Insert 3 calls to the Orange subroutine into the Blue subroutine |
|  | - Upload the program to the S2 |
|  | - Run the program |
|  | B: Modify SOS |
|  | - Change the time in the yellow sub routine from .25 seconds to .10 seconds and re-upload the program to Scribbler |
|  | - Record the results below |
|  | C: Optional: Morse Code Initials |
|  | - Use Morse Code to flash your initials |
|  | - Upload the program to the S2 and test |

Observations

Enter your observations for this activity here:

- What happened when you ran the program?

- Were you able to successfully complete the activity?     If not, why - what happened?

- Did the robot operate as you anticipated?   If not, what happened?

- What are the benefits of using subroutines?

- What happened when the time was changed from .25 seconds to .10 seconds?  Was the Orange Subroutine affected?  Why or why not?

- If you created the Morse Code Initials program, what was the hardest part?

- Optional - Insert a picture of the completed activity – Robot and Program Maker Screen